Packet Tracer – Comparing RIP and EIGRP Path Selection

- Topology

Objectives

Part 1: Predict the Path

Part 2: Trace the Route

Part 3: Reflection Questions

Scenario

PCA and PCB need to communicate. The path that the data takes between these end devices can travel through R1, R2, and R3, or it can travel through R4 and R5. The process by which routers select the best path depends on the routing protocol. We will examine the behavior of two distance vector routing protocols, Enhanced Interior Gateway Routing Protocol (EIGRP) and Routing Information Protocol version 2 (RIPv2).

Predict the Path

Metrics are factors that can be measured. Routing protocols are each designed to consider various metrics when considering which route is the best to send data along. These metrics include, hop count, bandwidth, delay, reliability, path cost, and more.

Consider EIGRP Metrics.

EIGRP can consider many metrics. By default, however, bandwidth and delay are used to determine best path selection.

Based on the metrics, what path would you predict data would take from PCA to PCB?

- RA, R1, R2, R3, RB, PCB____________________________________________________________________________________

Consider RIP Metrics.

What metric(s) are used by RIP? _ Hop count_________________

Based on the metrics, what path would you predict data would take from PCA to PCB?

- RA, R4, R5, RB, PCB____________________________________________________________________________________

Trace the Route

Examine the EIGRP Path.

On RA, view the routing table using the appropriate command. Which protocol codes are listed in the table and what protocols do they represent? C=connected and D=EIGRP________________________________________________

Trace the route from PCA to PCB.

What path does the data take? 64.100.0.254, 64.101.0.2, 64.101.0.6, 64.101.0.10, 64.101.0.14___________________________________________________________

How many hops away is the destination? 5 hops__________________

What is the minimum bandwidth on the path? _4mb/s_________________

Examine the RIPv2 Path.

You may have noticed that, while RIPv2 is configured, the routers ignore the routes that it generates, because they prefer EIGRP. Cisco routers use a scale called administrative distance and we need to change that number for RIPv2 in RA to make the router prefer the protocol.

For reference purposes, show the routing table of RA using the appropriate command. What is the first number between the brackets in each EIGRP route entry? ______90____________

Set the administrative distance for RIPv2 using the following commands. This forces RA to choose RIP routes over EIGRP routes.

RA(config)# router rip

RA(config-router)# distance 89

Wait a minute and show the routing table again. Which protocol codes are listed in the table and what protocols do they represent? _C, RIP___________________________________

Trace the route from PCA to PCB.

What path does the data take? 64.100.0.254, 64.102.0.2, 64.102.0.6, 64.102.0.14___________________________________________________________

How many hops away is the destination? _____4 hops_____________

What is the minimum bandwidth on the path? ___1.2kb/s_______________

What is the first number between the brackets in each RIP entry? ______89____________

Reflection Questions

What metrics does the RIPv2 routing protocol ignore? _ Everything but hops___________________________________________

How could this affect its performance? _ Answer will vary; RIP will ignore a faster bandwidth path_______________________________________________________

What metrics does the EIGRP routing protocol ignore? Hops.__________________

How could this affect its performance?

Answer will vary; It is conceivable that a packet could be dropped if it travels more hops than its TTL value allows._______________________________________________________________________________________

Which do you prefer for your own Internet access, lower hops or more bandwidth? _ Open ended answer.____________________

Is one routing protocol suitable for all applications? Why?

- _ Open ended answer.______________________________________________________________________________________

- _______________________________________________________________________________________

- _______________________________________________________________________________________

- Suggested Scoring Rubric

| Activity Section | Question Location | Possible Points | Earned Points |
| --- | --- | --- | --- |
| Part 1: Predict the Path | Step 1-b | 8 |  |
|  | Step 2-a | 8 |  |
|  | Step 2-b | 8 |  |
| Part 1 Total |  | 24 |  |
| Part 2: Trace the Route | Step 1-a | 8 |  |
|  | Step 1-b | 8 |  |
|  | Step 2-a | 8 |  |
|  | Step 2-c | 8 |  |
|  | Step 2-d | 8 |  |
|  | Step 2-e | 8 |  |
| Part 2 Total |  | 48 |  |
| Part 3: Reflection Questions | 1 | 7 |  |
|  | 2 | 7 |  |
|  | 3 | 7 |  |
|  | 4 | 7 |  |
| Part 3 Total |  | 28 |  |
| Total Score |  | 100 |  |